Enter the Inner Diameter in inches.362

Enter the cross-sectional diameter in inches.103

CS =

0.1030

Enter maximum o-ring compression (%)30

Enter minimum o-ring compression (%)20

Enter CS tolerance.003

Enter Groove diameter tolerance.01

Minimum bore diameter in inches =0.524588

Maximum bore diameter in inches =0.541988

Groove diameter is 0.379388